Homework 11 : Performance Analysis Methodology

Due after Session 37-39

Topic: Roofline Model & Profiling Practice

Q1 (Calculation) A GPU has a peak throughput of 312 TFLOPS (FP16 Tensor) and peak memory bandwidth of 2 TB/s.

(a) What is the arithmetic intensity threshold (ridge point) in FLOPS/Byte?

(b) A matrix multiplication kernel has arithmetic intensity of 200 FLOPS/Byte. Is it compute-bound or memory-bound? What is the theoretical maximum throughput in TFLOPS?

(c) An element-wise addition kernel (1 FLOP per element, 4 bytes read + 4 bytes written) has what arithmetic intensity? What is its theoretical throughput ceiling in TFLOPS?

Q2 (Hands-On, Required) Select a core FlagGems operator (e.g., LayerNorm or Softmax) and generate a complete performance report using Nsight Compute or torch.profiler. The report should include:

- Kernel execution time, achieved occupancy, memory throughput

- Roofline Model classification (compute-bound or memory-bound)

- At least one optimization suggestion

Submit the profiling report screenshot + your analysis (≤300 words).

Q3 (Short Answer) What is the core idea of auto-tuning in AI operator optimization? Compared to manual tuning, what are the main advantages and limitations of auto-tuning?

Q4 (Analysis) On a Roofline Model plot (x-axis: arithmetic intensity in FLOPS/Byte, y-axis: throughput in TFLOPS), qualitatively place the following four operators and label each as compute-bound or memory-bound:

(a) Large-scale matrix multiplication (e.g., GEMM, M=N=K=4096)

(b) Element-wise activation function (e.g., ReLU)

(c) Batch LayerNorm (large reduction dimension)

(d) Sparse matrix multiplication (sparsity=90%)

Briefly explain the rationale for each placement.

Q5 (Hands-On, Bonus) FlagPerf supports comparing kernel performance across configurations. Select a FlagGems operator and run it in both FP16 and BF16, comparing kernel execution time and numerical precision (using FP32 as ground truth, compute max absolute error). Submit a comparison table and your analysis.