Homework 3-en Memory Optimization

# Homework 3 : Memory Optimization

Due after Session 7-8

#### Topic: Matrix Multiplication & Memory Hierarchy

Q1 (Short Answer) In Tiled Matrix Multiplication, why is it necessary to tile the matrices? If each thread directly computes one element of the output matrix without tiling, what performance problem arises? Analyze from a memory bandwidth perspective.

Q2 (Short Answer) What is a Bank Conflict? Give an example of a common Shared Memory access pattern that causes bank conflicts, and describe a simple fix (e.g., padding).

Q3 (Analysis) Suppose you have a Reduction Kernel that sums an array of length N. Compare the performance of two strategies:

- Strategy A: Tree-based reduction within each thread block using Shared Memory, then atomic operations to merge block results.

- Strategy B: No Shared Memory; every thread directly uses tl.atomic_add to write to global memory.

Analyze from the perspectives of “global memory access count” and “atomic operation contention.”

Q4 (Calculation) Consider multiplying two square matrices A and B of dimension N×N in FP32 (4 bytes).

(a) Without tiling, how many bytes must be read from global memory to compute one output element? What is the total global memory read volume for the entire multiplication?

(b) With a tile size of T×T using Shared Memory caching, what does the total global memory read volume become? Derive the formula and compute the concrete value for T = 32, N = 4096.

Q5 (Short Answer) How do L1/L2 caches on modern GPUs differ in design from CPU caches? What is the relationship between the GPU’s L1 cache and Shared Memory (using NVIDIA Ampere as an example)? Why is understanding this helpful for writing efficient Triton kernels?