Homework 1: Conceptual Understanding

Due after Session 1–2

Topic: AI Hardware Landscape & Heterogeneous Programming Models

Q1 (Short Answer) Compare the design philosophies of CPUs, GPUs, and NPUs from two dimensions: instruction-level parallelism and data-level parallelism. Describe each processor type in 2–3 sentences.

Q2 (Short Answer) What specific challenges does the slowdown of Moore’s Law pose for AI system software? List at least three aspects and briefly explain which one(s) FlagOS aims to address.

Q3 (Short Answer) Draw a layered diagram of the AI system software stack (from upper-level applications/frameworks down to hardware) with at least 5 layers. Annotate where each FlagOS ecosystem component (FlagGems, FlagTree, FlagScale, FlagCX, FlagPerf) belongs.

Q4 (Open-Ended) MLSysBook notes that modern GPUs like the H100 provide ~3 TB/s memory bandwidth, yet loading the weights of a 70B-parameter model alone takes ~47 ms. Using the SIMT execution model, explain why “faster compute units” do not always yield proportional end-to-end speedups. Where might the bottleneck lie?

Q5 (Short Answer) What is a “Warp” in a GPU? What happens when threads within a warp execute conditional branches (if-else) — i.e., what is Warp Divergence? How does this affect AI operator performance?